Product Manual

Configuration-specific Monoclonal Antibody Based

# Rac Activation Assay Kit

Catalog Number： CR-W 80501

## 30 assays

Table of Content

Product Description                                                                                     3

Assay Principle                                                                                            3

Kit Components                                                                                           3

Storage                                                                                                          4

Materials Needed but Not Supplied                                                          4

Reagent Preparation                                                                                    4

Sample Preparation                                                                                     5

Assay Procedure                                                                                          6

Example of Results                                                                                      8

FOR RESEARCH USE ONLY.

NOT FOR USE IN DIAGNOSTIC PROCEDURES

# Product Description

Small GTPases are a super-family of cellular signaling regulators. Rac belongs to the Rho sub-family of GTPases that regulate cell motility, cell division, and gene transcription. GTP binding increases the activity of Rac, and the hydrolysis of GTP to GDP renders it inactive.

Currently the activation of Rac proteins is assayed with the binding of GTP-bound Rac to the p21-binding domain (PBD) of p21-activated protein kinase (PAK).    This method is based on the observation that the active, GTP-bound Rac could bind to the PBD of PAK.    However, the reproducibility of this method is poor.    This is partially due to the relatively quick hydrolysis of GTP to GDP during the assay procedure, and the low binding affinity of PBD to Rac-GTP.

NewEast  Biosciences  Rac  Activation  Assay  Kit  is  based  on  the  configuration-specific  monoclonal antibody  that   specifically  recognizes   Rac-GTP,  but   not  Rac-GDP.     Given  the  high   affinity  of monoclonal antibodies to their antigens, the activation assay could be performed in a much shorter time. This assay provides the reliable results with consistent reproducibility.

These anti-Rac-GTP monoclonal antibody can also be used to monitor the activation of Rac in cells and in tissues by immunohistochemistry.

NewEast  Biosciences Rac Activation Assay Kit provides  a  simple  and  fast method to monitor the activation of Rac. Each kit provides sufficient quantities to perform 20 assays.

# Assay Principle

NewEast  Biosciences  Rac  Activation  Assay  Kit  bases  on  the  configuration-specific  anti-Rac-GTP monoclonal antibody to measure the active Rac-GTP levels, either from cell extracts or from in vitro GTPγS loading Rac activation assays.    Briefly,  anti-active Rac mouse monoclonal  antibody will be incubated with cell lysates containing Rac-GTP.    The bound active Rac will then be pulled down by protein A/G agarose. The precipitated active Rac will be detected by immunoblot analysis using anti-Rac rabbit polyclonal antibody.

# Kit Components

1.   Anti-active Rac, Mouse Monoclonal Antibody (Catalog No. 26903): One vial – 22 µL (1 mg/ml) in PBS, pH 7.4, containing 50% glycerol and 0.05% sodium azide.    This antibody specifically recognizes Rac-GTP from all vertebrates.

2.   Protein A/G Agarose (Catalog No. 30301): One vial – 400 µL of 50% slurry.

3.   5X Assay/Lysis Buffer (Catalog No. 30303): One bottle – 30 mL of 250 mM Tris-HCl, pH 8, 750

mM NaCl, 50 mM MgCl2, 5 mM EDTA, 5% Triton X-100.

4.   Anti-Rac, Rabbit Polyclonal Antibody (Catalog No. 21003): One vial – 22 µL (1 mg/ml) in PBS, pH 7.4, contained 50% glycerol.

5.   100 X  GTPγS  (Catalog No. 30302): One vial –100  μl at  10 mM, use  5 µL of GTPγS for GTP-labeling of 0.5 mL of cell lysate.

6.   100  X  GDP  (Catalog  No.  30304):  One  vial  –100  μl  at  100  mM,  use  5  µL  of GDP  for GDP-labeling of 0.5 mL of cell lysate.

# Storage

º

Store all kit components at 4 C until their expiration dates.

# Materials Needed but Not Supplied

1.     Stimulated and non-stimulated cell lysates

2.     Protease inhibitors

3.     4 °C tube rocker or shaker

4.     0.5 M EDTA, pH8.0 5.      1 M MgCl2

6.     2X reducing SDS-PAGE sample buffer

7.     Electrophoresis and immunoblotting systems

8.     Immunoblotting wash buffer such as TBST (10 mM Tris-HCl, pH 7.4, 0.15 M NaCl, 0.05% Tween-20)

9.     Immunoblotting blocking buffer (TBST containing 5% Non-fat Dry Milk or 3% BSA)

10.   PVDF or nitrocellulose membrane

11.   Secondary Antibody

12.   ECL Detection Reagents

Reagent Preparation

• 1X Assay/Lysis Buffer: Mix the 5X Stock briefly and dilute to 1X in deionized water. Just prior to

usage, add protease inhibitors such as 1 mM PMSF, 10 µg/mL leupeptin, and 10 µg/mL aprotinin.

# Sample Preparation

Adherent Cells

1.   Culture cells (one 10-cm plate, ~ 107  cells) to approximately 80-90% confluence. Stimulate cells with activator or inhibitor as desired.

2.   Aspirate the culture media and wash twice with ice-cold PBS.

3.   Completely remove the final PBS wash and add ice-cold 1X Assay/Lysis Buffer to the cells (0.5

- 1 mL per 10 cm tissue culture plate).

4.   Place the culture plates on ice for 10-20 minutes.

5.   Detach the cells from the plates by scraping with a cell scraper.

6.   Transfer the lysates to appropriate size tubes and place on ice.

7.   If nuclear lysis occurs, the cell lysates may become very viscous and difficult to pipette. If this occurs,  lysates  can  be  passed  through  a  27½-gauge  syringe  needle  3-4  times  to  shear  the genomic DNA.

8.   Clear the lysates by centrifugation for 10 minutes (12,000 x g at 4 °C).

9.   Collect the supernatant and store samples (~1-2 mg of total proteins) on ice for immediate use, or snap freeze and store at - 70 °C for future use.

Suspension Cells

1.   Culture cells and stimulate with activator or inhibitor as desired.

2.   Perform a cell count, and then pellet the cells by centrifugation.

3.   Aspirate the culture media and wash twice with ice-cold PBS.

4.   Completely remove the final PBS wash and add ice-cold 1X Assay/Lysis Buffer to the cell pellet (0.5 – 1 mL per 1 x 107cells).

5.   Lyse the cells by repeated pipetting.

6.   Transfer the lysates to appropriate size tubes and place on ice.

7.   If nuclear lysis occurs, the cell lysates may become very viscous and difficult to pipette. If this occurs,  lysates  can  be  passed  through  a  27½-gauge  syringe  needle  3-4  times  to  shear  the genomic DNA.

8.   Clear the lysates by centrifugation for 10 minutes (12,000 x g at 4 °C).

9.   Collect the supernatant and store samples on ice for immediate use, or snap freeze and store at -

70 °C for future use.

In vitro GTPγS/GDP Protein Loading for positive and negative controls

Note: In vivo stimulation of cells will activate approximately 10% of the available Rac, whereas in vitro GTPγS protein loading will activate nearly 90% of the Rac.

1,    Aliquot 0.5 ml of each cell extract to two microfuge tubes (or use 1 μg of purified Rac protein).

2,    To each tube, add 20 µl of 0.5 M EDTA (to 20 mM final concentration).

3,    Add 5 µl of 100 X GTPγS (to 100 µM, final concentration) to one tube (positive control).

4,    Add 5 µl of 100 X GDP (to 1 mM, final concentration) to the second tube (negative control).

5,    Incubate the tubes at 30°C for 30 minutes with agitation.

6,    Stop loading by placing the tubes on ice and adding 32.5 µl of 1 M MgCl2  (to 60 mM, final concentration).

# Assay Procedure

I. Active Rac Pull-Down Assay

1.   Aliquot 0.5 – 1 mL of cell lysate to a microcentrifuge tube.

2.   Adjust the volume of each sample to 1 mL with 1X Assay/Lysis Buffer.

3.   Add 1 μl anti-active Rac monoclonal antibody to the tube.

4.   Thoroughly resuspend the protein A/G Agarose bead slurry by vortexing or titurating.

5.   Quickly add 20 µL of resuspended bead slurry to each tube.

6.   Incubate the tubes at 4 °C for 1 hour with gentle agitation.

7.   Pellet the beads by centrifugation for 1 min at 5,000 x g.

8.   Aspirate and discard the supernatant, making sure not to disturb/remove the bead pellet.

9.   Wash the bead 3 times with 0.5 mL of 1X Assay/Lysis Buffer, centrifuging and aspirating each time.

10. After the last wash, pellet the beads and carefully remove all the supernatant.

11. Resuspend the bead pellet in 20 µL of 2X reducing SDS-PAGE sample buffer.

12. Boil each sample for 5 minutes.

13. Centrifuge each sample for 10 seconds at 5,000 x g.

II. Electrophoresis and Transfer

1.   Load 15 µL/well of pull-down supernatant to a polyacrylamide gel (17%). Also, it’s

recommended to include a pre-stained MW standard (as an indicator of a successful transfer in step 3).

2.   Perform SDS-PAGE following the manufacturer’s instructions.

3.   Transfer the gel proteins to a PVDF or nitrocellulose membrane following the manufacturer’s instructions.

III. Immunoblotting and Detection (all steps are at room temperature, with agitation)

1. Following the electroblotting step, immerse the PVDF membrane in 100% Methanol for 15 seconds, and then allow it to dry at room temperature for 5 minutes.

Note: If Nitrocellulose is used instead of PVDF, this step should be skipped.

2.   Block the membrane with 5% non-fat dry milk or 3% BSA in TBST for 1 hr at room temperature with constant agitation.

Incubate the membrane with anti-Rac polyclonal antibody, freshly diluted 1:50~1000 (depending on the amount of Rac proteins in your samples) in 5% non-fat dry milk or 3% BSA/TBST, for

1-2 hr at room temperature with constant agitation or at 4oC overnight.

3.   Wash the blotted membrane three times with TBST, 5 minutes each time.

4.   Incubate the membrane with a secondary antibody (e.g. Goat Anti-Rabbit IgG, HRP-conjugate), freshly diluted 1:1000 in 5% non-fat dry milk or 3% BSA/TBST, for 1 hr at room temperature with constant agitation.

5.   Wash the blotted membrane three times with TBST, 5 minutes each time.

6.   Use the detection method of your choice such as ECL.

# Example of Results

The following figure demonstrates typical results seen with NewEast Biosciences Rac Activation Assay Kit. One should use the data below for reference only.

Rac activation assay.    MEF cells were treated with (lane 2) or without (lane 1) PDGF.     Cell lysates were  incubated  with  an  anti-active  Rac  monoclonal  antibody  (Cat.  #  26903)  (top  panel).    The precipitated  active  Rac  was  immunoblotted  with  an  anti-Rac  rabbit  polyclonal  antibody  (Cat  # 21003).    The bottom panel shows the Western blot with anti-Rac of the cell lysates used (5% of that used in the top panel).

科博瑞（武汉）生物科技有限公司

TEL：027-87615885  15972062685

- mail：3636901710@qq.com

- Web:www.coburybio.com